PROJECT EXPLAINER  |  PYTHON SIMULATION + WEB DASHBOARD

# Microgrid RL

Executive & Technical Project Guide

A single, plain-language walkthrough of the energy-management problem, simulator, trained agents, benchmark, measured results, dashboard, and how to run the project.

In one sentence This project trains and compares three reinforcement-learning controllers that decide when a home battery should charge or discharge, compares them with a mathematical schedule, and displays results in a browser dashboard.

| Project at a glance | Verified state in this workspace |
| --- | --- |
| Main task | Choose hourly battery power for a 24-hour solar-and-load profile under time-of-use prices. |
| Controllers | SAC, robust SAC (R-SAC), PPO; plus LP, Rule-Based, and NoOp comparisons. |
| Battery model | 13.5 kWh capacity; up to 5 kW charge/discharge; 10% SOC floor. |
| Latest training run | Pipeline completed. Existing SAC was reused; R-SAC and PPO were newly trained for 200,000 steps each. |
| Latest dashboard data | simulation.json was refreshed after training, with a separate 25-episode evaluation. |
| Input data used | Synthetic solar/load profiles; neither expected CSV file was present at run time. |

Prepared from repository source, documentation, saved result files, training log, and exported dashboard JSON. Snapshot date: 18 October 2026.

# How to read this guide

Start with the short summary, then follow the system flow from input data to dashboard. The tables explain the physics and cost model, each learning method, and what the saved results say. The final sections explain how to rerun the work and where each part of the code lives.

| Section | What it answers |
| --- | --- |
| 1. Executive summary | What the project is and what the current results show. |
| 2. System flow | How data moves through training, evaluation, and the browser. |
| 3. Problem and simulator | What the battery can do and how reward is calculated. |
| 4. Data and forecast design | Which profiles are loaded, forecast vs actual, and what happened in this run. |
| 5. Agents and benchmark | How SAC, R-SAC, PPO, LP, and baselines work. |
| 6. Training, evaluation, results | How models were trained, metric meanings, and current outcomes. |
| 7. Dashboard | What the React site displays and how its data is generated. |
| 8. Runbook | Commands for training, exporting, and viewing the dashboard. |
| 9. Repository map and outputs | Where to find code, models, tables, and plots. |
| 10. Limits and next steps | What the experiment supports and what should improve before stronger claims. |

# 1. Executive summary

The project models a small residential microgrid with solar generation, household demand, a utility connection, and one battery. At each simulated hour, a policy sees the current state and a 24-hour forecast window, chooses a charging or discharging request, and receives a reward equal to negative operating cost.

It compares two standard reinforcement-learning algorithms (SAC and PPO) with R-SAC, which is SAC trained while solar/load reality is randomly perturbed. A linear-programming (LP) schedule is a benchmark. The project also includes a fixed rule and an idle-battery baseline.

The Python pipeline trains or loads agents, evaluates them, saves CSV summaries, creates PDF plots, and runs forecast-mismatch scenarios. A separate export script reruns a smaller evaluation and serializes chart inputs into simulation.json. The React/TypeScript app reads that JSON and renders an interactive day replay, comparisons, and a robustness view.

The current evidence, stated plainly On the saved 100-episode evaluation, every strategy has negative “profit” because this field is defined as minus total cost. The LP is best on the clean synthetic profile; R-SAC is the strongest learned agent. In the saved 30% forecast-error sweep, the LP still has a slightly better mean outcome than R-SAC. These results are for one synthetic daily profile, not evidence of general performance across real homes or seasons.

# 2. End-to-end system flow

| 1 Profiles | 2 Simulator | 3 Policies | 4 Score | 5 Dashboard |
| --- | --- | --- | --- | --- |
| CSV if available; otherwise seeded synthetic 24-hour solar/load. Add fixed TOU prices. | Gymnasium applies battery limits, efficiency, ramping, grid balance, and hourly cost. | SAC / R-SAC / PPO choose one continuous action each hour. Rule-Based and NoOp are baselines. | LP benchmark, 100-episode evaluation, CSV/PDF reports, and a 0–30% noise sweep. | export_web_data.py writes simulation.json; Vite/React fetches it and renders charts. |

Typical pipeline order: setup data → train/load agents → evaluate → generate plots → stress-test forecasts → export JSON → serve the dashboard.

## A single simulated hour

| Step | What happens |
| --- | --- |
| Observe | Policy receives SOC, forecast net demand and price, time-of-day encoding, recent history, last forecast error, and the next 24 forecast feature rows. |
| Act | One scalar action from −1 to +1 is scaled to a battery request of −5 to +5 kW. Positive means charge; negative means discharge. |
| Constrain | Simulator applies the 2.5 kW/hour ramp limit, battery SOC bounds, power limit, and charging/discharging efficiency. |
| Settle | Grid power is household net demand plus battery charging power. The model applies import cost/export credit, demand penalty, and degradation cost. |
| Learn / record | Reward is negative hourly cost. At hour 24, a terminal SOC penalty is added if the battery is away from its 50% target. |

# 3. The problem and the simulator

## What is being controlled?

The controller shifts energy across the day. It may charge during low-price hours or solar surplus, then discharge when household demand is expensive. It must respect battery limits and return near its starting charge at the end of the simulated day.

## Plant settings in the current run

| Setting | Value | Meaning |
| --- | --- | --- |
| Battery capacity | 13.5 kWh | Maximum stored energy; described as Powerwall-like. |
| SOC range | 10%–100% | Minimum reserve and maximum battery charge. |
| Power limit | ±5.0 kW | Maximum hourly charge/discharge request. |
| Efficiency | 95% each direction | About 90.25% round-trip efficiency (0.95 × 0.95). |
| Ramp limit | 2.5 kW per hour | Battery power cannot change by more than this between decisions. |
| Grid threshold | 10 kW | Hourly import above this incurs the configured peak penalty. |
| Peak penalty | $0.50 per kW above threshold | Applied to each hour’s exceedance; not a monthly utility demand-charge model. |
| Battery wear | $0.02 per kWh throughput | Adds a cost when stored energy changes. |
| End-of-day target | 50% SOC | Environment penalizes a final SOC away from 50%. |

## Tariff schedule

| Hours | Band | Import price | Export credit |
| --- | --- | --- | --- |
| 00:00–05:59 | Off-peak | $0.08/kWh | 40% of import price |
| 06:00–13:59 | Mid-peak | $0.15/kWh | 40% of import price |
| 14:00–20:59 | On-peak | $0.28/kWh | 40% of import price |
| 21:00–23:59 | Mid-peak | $0.15/kWh | 40% of import price |

## Cost and reward in everyday terms

Each hour combines energy cost, any import above the 10 kW threshold, and battery wear. Imports are charged at the time-of-use price; exports earn a smaller credit. On the final hour, a one-time penalty of $50 × the absolute difference between final SOC and 50% SOC is added. Learning reward is the negative of this cost.

Reading the numbers The CSV column called “Mean Profit ($)” is actually −total cost. A value of −$0.75 means modeled cost of about $0.75 for that day; −$0.30 is better because it is closer to zero. The current run does not produce positive cash profit.

## The 131-number observation

| Part | Size | Contents |
| --- | --- | --- |
| Current state | 7 | SOC; net power; price; sine/cosine of hour; previous battery power; last actual-vs-forecast net-power error. |
| Recent history | 4 | Rolling buffer of recent net-demand values. |
| Forecast window | 120 | 24 hours × 5 values: net power, price, solar, and sine/cosine of time. |
| Total | 131 | 7 + 4 + 120. Forecast features make up over 90% of the input. |

Forecast and actual profiles are separate. Look-ahead features use forecasts; battery/grid physics use actuals. In standard evaluation they match. In robustness evaluation, solar and load actuals are perturbed while the policy receives the unperturbed forecast and current feedback through history/error features.

# 4. Data and forecast design

## Accepted input files

| Input | Expected columns / handling |
| --- | --- |
| electricityConsumptionAndProductioction.csv | Generic CSV with DateTime, Consumption, Solar. Consumption and Solar are assumed watts, converted to kW, and grouped into complete 24-row days. |
| PecanStreet_10_Homes_1Min_Data.csv | Pecan Street CSV with Datetime (UTC), Home ID, Circuit, Value. Solar and Main Panel circuits are averaged hourly; complete days become profiles. |
| No file found | Seeded synthetic 24-hour solar and load arrays are generated. This is what the saved Oct 18 run used. |

When data exist, setup tries the generic file first, then Pecan Street. It selects the complete day with the most solar; on these real-data paths it multiplies solar by 5 to represent a larger PV system. The price schedule remains fixed.

## Synthetic profile used in this workspace

At the latest training/export time, neither named CSV was present. The project generated seeded solar (maximum about 5.11 kW, average about 3.00 kW) and load (average about 3.01 kW). This supports repeatable runs without private datasets, but ordinary SAC/PPO training and baseline evaluation see one daily shape.

## How forecast error is created

ScenarioGenerator applies multiplicative, correlated AR(1) noise and clips values to bounds. R-SAC training randomizes actual solar/load each episode, with noise drawn from 0% to 30% and correlation ρ = 0.9. Robustness evaluation uses the generator’s default correlation ρ = 0.8.

The robustness generator also makes a noisy price series, but current evaluators pass the original fixed tariff into the environment and LP. The saved robustness experiment perturbs solar and load, not electricity prices.

Why this matters Synthetic data can verify the code path and power the dashboard. It cannot establish that an agent will work well across homes, weather, tariffs, or battery systems. The saved evaluation episodes mostly vary initial SOC, not the daily profile.

# 5. Agents and the benchmark

| Policy | How it works here | Key implementation settings |
| --- | --- | --- |
| SAC | Soft Actor-Critic: off-policy actor-critic learns from replay and balances reward with exploration. | LR 3e−4; replay 50,000; batch 256; gamma .99; tau .005; MLP [256, 256]. |
| R-SAC | SAC with domain randomization: actual solar/load changes each episode while forecast stays nominal. | LR 1e−4; buffer 100,000; batch 512; gradient steps 2; learning starts 1,000; MLP [256,256,128]; noise up to 30%, ρ=.9. |
| PPO | Proximal Policy Optimization: on-policy clipped updates; observations are normalized, and saved normalization stats are needed for evaluation. | LR 3e−4; rollout 4,096; batch 128; 15 epochs; gamma .99; GAE .95; clip .2; separate [256,256,128] policy/value networks. |
| LP | SciPy HiGHS linear program uses the whole day’s profile to choose a schedule; perfect foresight in clean evaluation. | Hourly charge/discharge, grid import/export, peak-excess variables; power balance, SOC, terminal SOC, and ramp constraints. |
| Rule-Based | Fixed heuristic: charge on solar surplus; discharge when demand is positive and price is high, subject to SOC. | No learning; reads current normalized net power and price. |
| NoOp | Never moves the battery. | Idle baseline. |

## How to interpret the LP

The LP is an optimization benchmark, not another learned controller. In clean comparison it knows the full 24-hour profile and solves once, so it is expected to be hard to beat. In robustness testing, it plans against the nominal profile and the fixed schedule is executed against noisy solar/load. RL policies choose again every hour.

LP terminal SOC is constrained exactly to 50%; RL receives a penalty for ending away from 50%. Their objectives are designed to be similar, but code-level accounting details in Section 10 should be reconciled before close numerical comparisons.

# 6. Training, evaluation, and current results

## Training record in the saved log

| Agent | What this run did | Saved artifact |
| --- | --- | --- |
| SAC | Loaded an existing 200,000-timestep model; not retrained in this resume run. | results/sac_model.zip (saved Oct 16, 2026) |
| R-SAC | New 200,000-timestep training run; log reports about 102.0 minutes. | results/rsac_model.zip (saved Oct 18, 2026) |
| PPO | New 200,000-timestep training run; log reports about 7.4 minutes. | results/ppo_model.zip and results/ppo_vec_normalize.pkl (Oct 18) |

The total new training time recorded was about 109.3 minutes, excluding evaluation/plots/export. resume_training.py is a pipeline-resume helper: it loads and skips existing models; it does not automatically add steps to a loaded model. Use --force and an agent name to retrain it.

## Evaluation design

Main/resume evaluates 100 episodes. Starting SOC is swept evenly from 20% to 80%; each learned policy acts deterministically.

The ordinary evaluation uses the same selected 24-hour solar/load shape; starting SOC changes. LP is solved separately for each SOC.

Saved measures include mean, standard deviation, min/max, energy result, peak penalty, throughput, final SOC, and gap versus LP.

Gap = (LP profit − policy profit) ÷ |LP profit| × 100. Since profits are small negative costs, this percentage can look huge; also compare dollar values.

The exporter independently runs 25 episodes and a new 10-scenario robustness sweep. Dashboard numbers can differ from the 100-episode report.

## Saved 100-episode evaluation — results/evaluation_results.csv

| Policy | Mean profit (± standard deviation) | Plain reading |
| --- | --- | --- |
| LP | −$0.30 (± $0.17) | Best saved mean result on clean synthetic profile. |
| R-SAC | −$0.75 (± $0.20) | Best learned agent in this evaluation. |
| SAC | −$1.49 (± $0.19) | Below R-SAC on this profile. |
| PPO | −$3.69 (± $0.13) | Below SAC and R-SAC on this profile. |
| NoOp | −$8.33 (± $4.37) | Idle battery; variation mainly relates to terminal SOC cost. |
| Rule-Based | −$25.88 (± $1.25) | Poor result here; it often finishes near full and incurs terminal cost. |

All values are costs with a negative sign labelled as profit. Saved evaluation reports zero peak penalty for these episodes; outcome differences mainly come from energy, wear, and terminal SOC.

## Saved robustness sweep — results/robustness_summary.csv

| Policy | Mean at 0% noise | Mean at 30% noise | Degradation |
| --- | --- | --- | --- |
| PPO | −$3.88 | −$4.34 | 11.76% |
| SAC | −$1.60 | −$2.17 | 36.06% |
| R-SAC | −$0.75 | −$1.49 | 100.08% |
| LP | −$0.64 | −$1.31 | 104.12% |

In dollar outcome, LP remains slightly ahead of R-SAC at 30% noise in this saved sweep. PPO has the smallest relative degradation because the score compares each agent with its own clean baseline; PPO still has the worst absolute cost. “Robustness score” is just 100 minus absolute degradation, not a standard reliability metric.

Two result snapshots are present The pipeline CSV/log uses 100 episodes. Afterward, export_web_data.py ran with 25 episodes and wrote simulation.json. The dashboard describes the 25-episode snapshot, so some dashboard figures differ from the 100-episode CSV.

## What the current dashboard JSON says

| Policy | Dashboard mean profit (25 episodes) | Dashboard noise sweep (0% → 30%) |
| --- | --- | --- |
| LP | −$0.30 (± $0.18) | −$0.71 → −$1.33 |
| R-SAC | −$0.75 (± $0.21) | −$0.72 → −$1.23 |
| SAC | −$1.49 (± $0.20) | −$1.52 → −$2.06 |
| PPO | −$3.69 (± $0.13) | −$3.85 → −$4.36 |
| NoOp | −$8.55 (± $4.52) | Not included in exported noise sweep |
| Rule-Based | −$25.79 (± $1.42) | Not included in exported noise sweep |

In this newer JSON sweep, R-SAC is about $0.10 better than LP at 30% noise, whereas the pipeline’s earlier 10-scenario sweep had LP about $0.18 better. Both use only 10 scenarios at each noise level; the reversal shows the small-sample sensitivity. The dashboard’s “most robust” label is based on relative degradation (PPO in both snapshots), not the best absolute dollar result at 30% noise.

# 7. The interactive dashboard

The web app is a static results viewer built with React, TypeScript, Vite, and Recharts. It does not train or simulate in the browser. It fetches web/public/data/simulation.json and builds charts from that file.

| Dashboard area | What the viewer can do |
| --- | --- |
| Hero | Show headline LP/best-learned/robustness/tariff stats and an illustrative system schematic. |
| Replay | Choose an agent; scrub or play the 24-hour trajectory at several speeds; compare with an LP ghost. |
| Comparison | Toggle SOC/profit charts; inspect means, deviations, cost breakdown, and score table. |
| Robustness | Move a 0–30% slider; inspect saved noise curves, ranking, and a forecast/realization illustration. |
| System | Review the 131 observations, battery settings, tariff profile, and cost components. |

Profiles, summary, trajectories, per-episode values, and robustness curves all come from export_web_data.py. Rerun the exporter after changing models, data, configuration, or evaluation code.

# 8. Runbook: how to use the project

## A. Open the project and activate its environment (Windows PowerShell)

| Command | Purpose |
| --- | --- |
| cd 'D:\AIFS project\microgrid-rl-energy-management' | Move to repository root. |
| .\venv\Scripts\Activate.ps1 | Use project Python with gymnasium and ML dependencies. |

If the environment does not exist yet, create it and install dependencies:

Command

python -m venv venv

.\venv\Scripts\Activate.ps1

python -m pip install -r requirements.txt

The error you saw Running export_web_data.py before activating venv used a Python installation without gymnasium. Activating .\venv\Scripts\Activate.ps1 and rerunning fixed the import; the export then completed.

## B. Train or reuse the agents

| Command | Behavior |
| --- | --- |
| python main.py | Full pipeline; trains SAC, R-SAC, PPO from scratch, then evaluates, plots, and runs robustness testing. |
| python resume_training.py | Loads existing model files and trains only missing agents; then evaluates/plots/stress-tests. |
| python resume_training.py --force rsac | Retrains R-SAC even if its ZIP exists; use sac or ppo similarly. |
| python main.py --demo | Short demonstration path with a small SAC run and quick evaluation. |

## C. Refresh and open the dashboard

| Command | Behavior |
| --- | --- |
| python export_web_data.py | Loads available models, runs 25-episode evaluation and fresh robustness sweep, captures a 50%-SOC day, rewrites web/public/data/simulation.json. |
| cd web | Enter front-end project. |
| npm install | Install React/Vite/Recharts dependencies. |
| npm run dev | Start local server; Vite prints address, usually http://localhost:5173. |
| npm run build | Type-check and make production bundle in web/dist. |

If the dashboard says data are missing, run the exporter from repository root and reload. If it looks stale, check the JSON modified time and refresh.

## D. Progress monitor

watch_training.py displays progress from a running main.py or resume_training.py log. Closing the monitor stops the display only; it does not itself stop training. The log’s PIPELINE COMPLETE line and saved artifacts are completion evidence.

# 9. Repository map and saved outputs

## Core Python files

| File | Role |
| --- | --- |
| main.py | Primary orchestration: profile selection, configuration, scratch training, evaluation, plots, robustness; also --demo. |
| resume_training.py | Loads existing agents, trains missing/forced models, repeats evaluation and robustness reporting. |
| microgrid_env.py | Gymnasium state/action spaces, battery physics, reward, forecast noise, and optional DynamicMicrogridEnv. |
| data_loader.py | Pecan Street/generic CSV loading, profile preparation, and fixed time-of-use/feed-in prices. |
| rl_agents.py | SAC/PPO/R-SAC factories, training callback, model persistence, and simple baselines. |
| lp_solver.py | SciPy HiGHS linear-program benchmark and fallback schedule. |
| evaluation.py | Episode loops, metrics, LP gaps, CSV summary/detail, PPO observation normalization. |
| robustness_test.py | Correlated-noise scenarios, LP/RL mismatch evaluation, scorecard and plots. |
| visualization.py | Main figures for SOC, profit, grid/battery power, ramping, price, gaps, efficiency, profiles, summary. |
| export_web_data.py | Writes fresh profiles, metrics, trajectories, and robustness curves to dashboard JSON. |
| watch_training.py | Console progress monitor using training logs and model timestamps. |

## Other analysis and visualization helpers

| File/group | Purpose |
| --- | --- |
| plot_results.py; plot_input_profiles.py | Supplementary thesis/report figures and input-profile figures. |
| plot_environment_diagram.py; create_multipanel_plot.py | Architecture diagram and multi-panel comparisons. |
| create_duck_curve.py; visualize_forecast_behavior.py | Illustrative duck curve and forecast-vs-actual figure. |
| regenerate_plots.py | Reloads available models, reruns a 50-episode evaluation, recreates main plots. |
| run_robustness_check.py | Standalone robustness run with saved agents and its own profile/configuration choices. |
| report_results.py; extract_profiles.py | Prints saved evaluation metrics or dumps current profile arrays. |

## Important saved outputs

| Path | What it contains |
| --- | --- |
| results/sac_model.zip | SAC model; reused in latest resume run. |
| results/rsac_model.zip | R-SAC model saved by latest training run. |
| results/ppo_model.zip | PPO model weights. |
| results/ppo_vec_normalize.pkl | Normalization state needed to reproduce PPO input handling. |
| results/evaluation_results.csv | 100-episode summary from completed pipeline. |
| results/evaluation_episodes.csv | Episode-level costs, SOC, throughput, grid totals, and metrics. |
| results/robustness_summary.csv; robustness_detailed.csv | Noise-sweep scorecard and scenario outcomes. |
| results/01_*.pdf … results/13_*.pdf | Comparison, power, cost, profile, efficiency, and robustness figures. |
| web/public/data/simulation.json | Dashboard payload; exported after training in a 25-episode run. |

CSV inputs and model binaries are ignored by .gitignore; private datasets are not included. Preserve model ZIPs and the PPO normalization file if they must be kept.

# 10. What the current results do not prove

Real-world performance. The latest data are synthetic. No physical grid, inverter, battery, weather service, or home is controlled; this is offline simulation.

Generalization across days. Ordinary SAC/PPO training and clean evaluation use one selected 24-hour profile. DynamicMicrogridEnv exists, but main.py does not use it to sample diverse days.

Robustness across all signals. Stress tests perturb solar and load; generated price noise is not used. The robustness helper uses ρ=.8; R-SAC training uses ρ=.9.

A definitive ranking from a small sweep. There are 10 scenarios at each noise level and one profile. Dashboard export runs a separate sweep.

A standard meaning for robustness score. It is an ad-hoc relative-degradation transformation; a smaller score does not necessarily mean better absolute cost.

Perfectly aligned LP accounting without review. Check first-hour ramping and degradation throughput/reporting before treating close numerical gaps as definitive.

## Recommended upgrades for stronger research claims

Train on many daily profiles, hold out different days/homes/seasons, and report generalization on those unseen profiles.

Use matched seeds, initial SOCs, scenarios, and tariffs; report per-scenario outcomes and uncertainty intervals.

Use noisy price series consistently if price forecast error is part of the intended experiment.

Reconcile LP and environment details, including first-hour ramping and degradation throughput/reporting, then rerun every output.

Report modeled operating cost beside the negative-profit display and state tariff/battery assumptions.

For live control, add a deployment/safety layer and validate using hardware or a trusted external simulator before connecting physical equipment.

# 11. Short glossary

| Term | Plain meaning |
| --- | --- |
| SOC | State of charge: stored battery energy divided by capacity. |
| kW / kWh | kW is power; kWh is energy over time. |
| TOU | Time-of-use tariff; electricity price changes over the day. |
| Forecast / actual | Forecast is what the policy is told; actual is used for physical outcomes. |
| SAC | Soft Actor-Critic, an off-policy reinforcement-learning method. |
| PPO | Proximal Policy Optimization, an on-policy reinforcement-learning method. |
| R-SAC | SAC trained with randomized forecast-to-actual mismatch. |
| LP | Linear program; constrained mathematical optimizer used as a benchmark. |
| AR(1) | Noise process where this hour’s error is correlated with the previous hour’s error. |
| Throughput | Energy moved into/out of the battery; align the exact accounting convention across LP/environment. |

# Evidence used for this guide

Checked against README.md and web/README.md; core Python modules; React/TypeScript components; requirements.txt and web/package.json; resume_training.log; saved model files; results/evaluation_results.csv; results/robustness_summary.csv; and web/public/data/simulation.json. The training log ends with PIPELINE COMPLETE. Model ZIPs and PPO normalization exist, and JSON was written after the pipeline.

Snapshot described: 18 October 2026. Changing data, code, model files, or configuration may change the results and dashboard output.